Imagine a warehouse has an FX9600 at the exit gate. Every carton has an RFID tag. The business wants three things: read cartons, monitor reader health, and control/configure the reader remotely.

The overall flow is:

RFID Tag

↓

RFID Radio

↓

Radio Control

↓

Reader Gateway

↓

MQTT / REST / HTTP

↓

Cloud or Local Application

Now let us map each part.

### Management Interface

Suppose the administrator wants to change reader settings.

Example:

"Change the reader configuration."

or:

"Update the firmware."

The application sends a request:

Application

↓

Management Interface

↓

Reader Gateway

↓

Reader Manager

↓

Configuration is changed

Then the reader sends a response:

Request:

Set configuration

Response:

Configuration updated successfully

That is why it is called synchronous.

Think:

Request → Response

So:

Management Interface = configure and manage the reader itself.

Typical examples:

Get configuration

Set configuration

Firmware update

Network configuration

Application configuration

### Management Events Interface

Now imagine nobody asks the reader anything.

But suddenly the reader CPU becomes very high.

The reader itself sends:

WARNING:

CPU utilization = 95%

Or every 60 seconds it sends:

Heartbeat

Temperature: 45°C

CPU: 35%

RAM: 52%

Status: Healthy

Nobody requested it.

That is why this is asynchronous.

Reader

↓

event happens

↓

Reader automatically sends event

↓

Application

So:

Management Events Interface = the reader proactively reports health, status, warnings, errors, GPIO events, and heartbeat information.

The difference is very important:

Management Interface

Application asks → Reader responds

Management Events Interface

Reader sends → without application asking

### Control Interface

Now the warehouse operator wants to start reading cartons.

The application sends:

START RFID READING

The flow is:

Application

↓

Control Interface

↓

Reader Gateway

↓

Radio Control

↓

RFID Radio starts scanning

Later:

STOP RFID READING

or:

Change RFID mode

So:

Control Interface = controls the RFID-reading operation.

A simple way to separate Management and Control is:

Management

= configure the reader

Control

= tell the RFID operation what to do

For example:

Management:

Configure MQTT broker

Control:

Start inventory

### Data Interface

Now the antenna detects a carton.

Suppose it reads:

EPC: BOX123

Antenna: 2

RSSI: -46

Timestamp: 10:30:20

The flow becomes:

RFID Tag

↓

RFID Radio

↓

Radio Control

↓

Reader Gateway

↓

Data Interface

↓

MQTT / HTTP

↓

Warehouse Application

So:

Data Interface = sends RFID tag-read information to external applications.

For example, your warehouse application receives:

BOX123 exited Gate 4

## Now understand the internal components

The interfaces are the external-facing side.

The reader components are the internal working side.

Think of the FX9600 like a small company.

FX9600

Reader Gateway

= receptionist

Reader Manager

= administrator

Radio Control

= RFID specialist

Reader Webserver

= local front desk / web UI

### Reader Gateway

This is the main communication bridge.

Suppose AWS sends:

Start reading

Reader Gateway receives it.

It then decides where that request needs to go.

AWS

↓

Reader Gateway

↓

Radio Control

If the request is:

Change system configuration

then:

Application

↓

Reader Gateway

↓

Reader Manager

It also works in the opposite direction.

When RFID tags are detected:

Radio Control

↓

Reader Gateway

↓

MQTT

↓

Cloud

So:

Reader Gateway connects the internal reader components to the outside world.

### Radio Control

This component deals with the RFID radio.

Suppose you configure:

Antenna 1

Power = 30 dBm

Session = S1

Start inventory

Radio Control applies those settings to the RFID radio.

Then:

RFID Radio

↓

Tag detected

↓

Radio Control receives tag

↓

Reader Gateway

So:

Radio Control = controls the physical RFID-reading process.

### Reader Manager

Suppose you want to:

Change hostname

Configure network

Update reader settings

Manage applications

Those are reader-management operations.

The Reader Manager performs those operations.

Management request

↓

Reader Gateway

↓

Reader Manager

↓

Reader configuration

So:

Reader Manager = manages reader configuration and system-level operations.

### Reader Webserver

Suppose your laptop and FX9600 are on the same network.

You open:

https://192.168.x.x

You see the reader's web interface.

That interface is provided by the Reader Webserver.

If Local REST is being used:

Local Application

↓

REST API

↓

Reader Webserver

↓

Reader Gateway

↓

appropriate component

So:

Reader Webserver = provides local web access and handles local REST requests.

## Put everything into one real sequence

At 9:00 AM, warehouse operations begin.

First, your application configures the reader:

Application

↓

Management Interface

↓

Reader Gateway

↓

Reader Manager

"Use these settings."

Then your application says:

Start inventory

Flow:

Application

↓

Control Interface

↓

Reader Gateway

↓

Radio Control

↓

RFID Radio

A carton passes:

BOX123

The radio detects it:

RFID Tag

↓

Radio

↓

Radio Control

↓

Reader Gateway

↓

Data Interface

↓

Cloud

Meanwhile the reader automatically sends:

Heartbeat

CPU 25%

RAM 40%

Temperature 42°C

through:

Management Events Interface

So all four interfaces have different responsibilities:

| Interface | Main question |
| --- | --- |
| Management | How should the reader be configured? |
| Management Events | How is the reader doing? |
| Control | What should the RFID reader do now? |
| Data | What RFID tags did the reader detect? |

And internally:

Reader Gateway

= communication bridge

Reader Manager

= manages reader configuration

Radio Control

= manages RFID reading

Reader Webserver

= local web/REST access

The single most important architecture to remember is:

EXTERNAL SYSTEM

Cloud / Local Application

↑ ↓

┌──────────────┼───────────────┐

│              │               │

Management       Control           Data

│              │               ↑

└──────────────┼───────────────┘

↓

Reader Gateway

/           \

↓             ↓

Reader Manager    Radio Control

↓

RFID Radio

↓

Antenna

↓

RFID Tags

Reader also sends:

Management Events ───────────────→ Application

If you understand this diagram, you have understood the basic system architecture of Zebra IoT Connector.

Sure. The main thing to understand is that deployment mode means: where each IoT Connector interface connects.

You already know the main interfaces:

Management

Control

Data

Management Events

Now deployment answers:

Do these interfaces connect to the cloud, stay inside the local network, or split between both?

Here is the easiest way to see it.

| Deployment mode | Where interfaces connect |
| --- | --- |
| Pure Cloud | Everything goes to cloud services |
| Local | Everything stays inside the local network |
| Hybrid | Some go to cloud, some stay local |

### Pure Cloud Deployment

Scenario: your company wants to manage the FX9600 from AWS.

You may have:

FX9600

│

├── Management ─────→ AWS

├── Control ─────────→ AWS

├── Data ────────────→ AWS

└── Management Events → AWS

Suppose a carton passes the reader.

RFID Tag

↓

FX9600

↓

Data Interface

↓

AWS IoT Core

↓

Warehouse Cloud Application

And your cloud application can also send:

Start reading

Stop reading

Change configuration

back to the reader.

So Pure Cloud means:

The cloud is the main place where the reader is managed, controlled, monitored, and where tag data is received.

AWS IoT Core and Azure IoT Hub are examples of this model.

### Local Deployment

Now imagine the customer does not want the reader communicating over the Internet.

Everything runs inside the warehouse network.

For example:

Warehouse Local Network

FX9600

│

├── Management ─→ Local server

├── Control ─────→ Local server

├── Data ────────→ Local server

└── Events ──────→ Local server

The reader can still communicate using technologies such as:

MQTT

REST

HTTP/HTTPS

The difference is not the protocol.

The difference is where the endpoint lives.

For example:

FX9600

↓

Local MQTT Broker

↓

Warehouse Application

No cloud is required.

### Local Deployment with MQTT

Imagine the warehouse already runs an MQTT broker.

Then you can configure:

FX9600

├── Management → MQTT Broker

├── Control → MQTT Broker

└── Data → MQTT Broker

Your application also connects to that broker.

So:

Application

↕

MQTT Broker

↕

FX9600

For example, the application publishes:

start inventory

The reader receives it.

Later the reader publishes:

Tag EPC = ABC123

Your application consumes it.

So MQTT becomes the messaging middleman.

### Local Deployment with HTTP / REST

Now imagine the customer does not want MQTT.

Management and control can happen directly through REST APIs.

For example:

Local Application

↓

REST API

↓

FX9600

Your application might send something conceptually like:

PUT configuration

or:

START reading

Tag data can be pushed by the reader to a local HTTP server:

RFID Tag

↓

FX9600

↓

HTTP POST

↓

Local Web Server

So this combination may look like:

Management → REST

Control → REST

Data → HTTP POST

### Mixed Local Deployment

This is important because the interfaces do not all have to use exactly the same protocol.

For example:

Management → REST

Control → REST

Data → MQTT

Everything is still local.

That is why I called it Mixed Local Deployment.

The important point is:

Local describes the location of the system, not one specific communication protocol.

### Hybrid Deployment

Now imagine your company wants centralized cloud management, but tag data is very large and must stay inside the warehouse.

You can do this:

Cloud

↑

Management

Control

FX9600

↓

Data

↓

Local Warehouse Server

So:

Management → AWS

Control → AWS

Data → Local HTTP Server

This is Hybrid Deployment.

Some interfaces use cloud endpoints.

Others use local endpoints.

That is why it is called hybrid.

A practical reason could be:

Cloud:

central reader administration

Local:

high-volume RFID tag processing

### Why do we need "Choosing a Deployment Mode"?

Because users should not just see three architectures. They need to know which one fits their environment.

For example:

Choose Pure Cloud

when:

- cloud connectivity is available

- centralized remote management is wanted

- cloud applications consume RFID data

Choose Local

when:

- Internet access is unavailable or restricted

- data should remain inside the site

- local applications consume RFID data

Choose Hybrid

when:

- cloud management is useful

- but some data/services must remain local

So the whole deployment chapter is really answering one question:

Where should Management, Control, Data, and Management Events go?

And the three answers are:

Pure Cloud

All → Cloud

Local

All → Local network

Hybrid

Some → Cloud

Some → Local

That is the core concept you need before writing the deployment section of the guide.

Sure. The easiest way is to understand this as one setup journey.

Imagine you have an FX9600 reader and you want this result:

RFID Tag

↓

FX9600

↓

IoT Connector

↓

MQTT / HTTP

↓

Your application

To make that happen, the setup has a few clear stages.

### 1. Configuration Overview

This is the high-level process.

You first tell the reader:

Where can I send data?

↓

Create endpoint

Which interface should use that endpoint?

↓

Assign interface

Now activate the setup

↓

Start IoT Connector

So the three steps are:

1. Add endpoint

2. Assign interface to endpoint

3. Start IoT Connector

That is the core workflow.

### 2. Access IoT Connector Configuration

Before configuring anything, you need to open the reader's configuration page.

For example:

Laptop

↓

Browser

↓

Reader IP address

↓

Communication

↓

Zebra IoT Connector

↓

Configuration

This section answers:

Where do I configure IoT Connector?

You are not yet deciding MQTT or HTTP. You are just opening the correct UI.

### 3. Configure Endpoints

This is very important.

An endpoint means destination.

Think:

Where should the reader connect or send information?

Examples:

MQTT Broker

HTTP Server

AWS IoT Core

Azure IoT Hub

Suppose you want tag data to go to an HTTP server.

You create:

Endpoint Name: webhook

Endpoint Type: HTTP POST

URL: https://example-server

Now the reader knows:

“This destination exists.”

But it still does not know what information should go there.

That comes in the next step.

### 4. Configure Interfaces

An interface means type of communication or purpose.

You already learned these:

Management

Control

Management Events

Tag Data

Now you connect an interface to an endpoint.

Example:

MQTT endpoint: mosquitto

↑

├── Management

├── Control

└── Management Events

And:

HTTP endpoint: webhook

↑

└── Tag Data

So the easiest way to remember is:

Interface = WHAT

Endpoint  = WHERE

Example:

WHAT?

Tag Data

WHERE?

HTTP Server

Therefore:

Tag Data Interface

↓

HTTP Endpoint

↓

HTTP Server

### 5. Verify Interface Connection Status

After assigning the interfaces, you need to check whether the connection actually worked.

For example:

Management → Connected

Control → Connected

Tag Data → Connected

Or perhaps:

Tag Data → Connection failed

This answers:

Did the reader successfully connect to the configured destination?

So this is not configuration anymore. It is verification.

### 6. Start IoT Connector

After configuration is ready, you start the service.

Think of it like configuring a machine and then switching it on.

Endpoint configured

↓

Interfaces assigned

↓

Connection verified

↓

Start IoT Connector

Now the communication system becomes active.

One important distinction:

Start IoT Connector

≠

Start RFID tag reading

Starting IoT Connector means:

Enable the communication/service.

Starting RFID reading means:

Tell the RFID radio to begin scanning tags.

They are related, but not the same action.

### 7. Start RFID Reading

Once the IoT Connector is running, you can tell the reader:

START READING

For example through:

Web UI

or:

REST API

PUT /cloud/start

or:

MQTT control command

The internal flow is roughly:

Application

↓

Control Interface

↓

Reader Gateway

↓

Radio Control

↓

RFID Radio

↓

Antenna

↓

RFID Tags

Now tag reading begins.

### 8. Test with MQTT Client

Suppose you configured MQTT.

How do you know MQTT communication is working?

You use an MQTT client such as MQTTX.

The flow is:

FX9600

↓

MQTT Broker

↑

MQTTX

MQTTX connects to the same broker and subscribes to the configured topics.

Then you can see things such as:

Heartbeat

Reader status

Warnings

Management events

So MQTTX is mainly a test client here.

It is not part of IoT Connector itself.

### 9. Send Commands Through MQTT

Once MQTT communication is working, the MQTT client can also publish commands.

For example:

MQTTX

↓

Command topic

↓

MQTT Broker

↓

FX9600

Command:

start

Then the reader begins tag reading.

So now you have tested both directions:

Application → Reader

commands

and:

Reader → Application

events/data

### 10. Verify Tag Data

Finally, a tag passes in front of the antenna.

RFID Tag

↓

Antenna

↓

RFID Radio

↓

Reader

↓

Tag Data Interface

↓

Configured endpoint

If Tag Data was assigned to HTTP:

FX9600

↓

HTTP POST

↓

Webhook / HTTP Server

If Tag Data was assigned to MQTT:

FX9600

↓

MQTT Broker

↓

Application

Now you know the complete configuration is working.

So the full journey is:

Open IoT Connector UI

↓

Create endpoint

WHERE should communication go?

↓

Assign interfaces

WHAT should go there?

↓

Check connection

Did it connect?

↓

Start IoT Connector

Enable communication

↓

Start RFID Reading

Begin scanning tags

↓

Test MQTT / REST

Can I control and monitor it?

↓

Verify Tag Data

Did the data reach my destination?

That is why the IA separates these topics. Each section answers a different user question.